CAS Number: 55295-98-2 Trade name: Decoloring Agent Chemical name: PolyDCD; Dicyandiamide formaldehyde resin Features & Applications: Water Decoloring Agent is a quaternary ammonium cationic copolymer, it's dicyandiamide formaldehyde resin. it has excellent efficiency in decoloring, flocculating and COD removal. 1. The product is mainly used to decolor the effluent with high colority from dyestuff plant. It is suitable to treat waste water with activated, acidic and disperse dyestuffs. 2. It can also be used to treat waste water from textile industry and dye houses, pigment industry, printing ink industry and paper industry. 3. It also can be used in the production process of paper& pulp as retention agent Specifications: Appearance; Colorless or light-color sticky liquid; Light yellow or yellow sticky liquid; Colorless or light-color sticky liquid. Solid Content; ≥50.0. Viscosity(mpa.s 20℃); 30-1000; 5-500; 30-1000. PH(30% water solution); 2.0-5.0. Application method and notes: 1 .The product shall be diluted with 10-40 times water, then added to the wastewater directly. After stirring for several minutes, the clear water will be got by precipitation or air-floatation. 2. The optimized pH of the wastewater accepted is 6-10. 3. It is recommended to use this product with the inorganic flocculants to treat the effluent with high color and COD to reduce the operation cost. The order and proportion of agent dosage depend on the flocculation test and the effluent treatment process. 4. The product would show layer separation and become white at low temperature. There is no negative impact on the usage after mixing up. Package and storage: Store in dry and ventilated room, recommended temperature 5-30℃. The product is packed in 250kg/drum, or 1250kg/IBC. Shelf life: 12 months .